DC21X4.COM

Novell ODI16 Network Driver Installation Guide

Revision 1.0

December 1997

Intel

Disclaimer

This readme.doc as well as the software described in it is

furnished under license and may only be used or copied in accordance with

the terms of the license. The information in this manual is furnished for

informational use only, is subject to change without notice, and should

not be construed as a commitment by Intel Corporation. Intel Corporation

assumes no responsibility or liability for any errors or inaccuracies that

may appear in this document or any software that may be provided in

association with this document.

Except as permitted by such license, no part of this document may be

reproduced, stored in a retrieval system, or transmitted in any form

or by any means without the express written consent of Intel Corporation.

© 	Copyright Intel Corporation 1994, 1995, 1996, 1997.

All rights reserved.

©    Copyright Novell, Inc. 1994, 1995.

Trademarks

Ethernet is a registered trademark of Xerox Corp.

Novell and NetWare are registered trademarks of Novell, Inc.

UnixWare is a trademark of Unix System Laboratories, Inc., a wholly-owned subsidiary of Novell, Inc.

Microsoft, LAN Manager, Windows for Workgroups, and Windows NT are registered trademarks of Microsoft.

Unix is a registered trademark of AT&T Technologies, Inc.

SCO is a registered trademark of Santa Cruz Operation, Inc.

OS/2 is a registered trademark of International Business Machines Corporation.

Intel is a registered trademark of Intel Corporation.

Digital Semiconductor, DEC, DECnet, Digital, PATHWORKS,	ThinWire, and the DIGITAL logo are trademarks of Digital Equipment Corporation.

All other trademarks and registered trademarks are the property of their respective holders.

Contents

1. Overview	4

1.1 The DC21X4 Family of Products	4

1.1.1 DC21X4 Hardware	4

1.1.2 DC21X4 Software	4

2.   Purpose	2

3.   Supplied Files	2

4.   Preinstallation Hardware and Software Requirements	2

4.1 Hardware Requirements	3

4.2 Firmware Requirements	3

4.3 Software Revision Requirements	4

5. Installing 21X4 Based Hardware Boards	4

6. Novell NetWare DOS ODI 16Bit Client Driver	5

6.1 Driver Installation	5

6.1.1 Driver Installation for NetWare 3.11/3.12 clients	5

6.1.2 Driver Installation for NetWare 4.1/3.12 clients with VLMs	8

6.1.2.1 First Installation of NetWare 4.1/3.12 clients with VLMs	8

6.1.2.2 Driver Upgrade of NetWare 4.1/3.12 clients with VLMs	9

6.2 Driver Keywords	10

6.2.1 NET.CFG Conventions	11

6.2.2 DC21X4  Keywords	12

Novell NetWare Operating System Keywords	12

DMA	12

IRQ	13

MEM	13

PORT	13

BUS ID	14

SLOT	14

NODE ADDRESS	15

FRAME	15

PROTOCOL	16

Driver Specific Keywords	17

Transmit Threshold (TX_THRESHOLD)	19

Number of Transmit Buffers (TX_BUFFERS)	19

Number of Receive Buffers (RX_BUFFERS)	20

ERI	20

Configuration Latency Timer (CFLT)	20

Programmable Burst Length (PBL)	21

Cache Alignment  (CAL)	21

Memory Read Multiple (MRM)	22

Memory Read Line (MRL)	22

Memory Write Invalidate (MWI)	22

TURBO	23

TP	24

TP FULL DUPLEX	24

TP NO LINK TEST	24

BNC	24

AUI	25

T4	25

LINE_SPEED	25

AUTOSENSE	26

MULTIA	27

GENERAL_PURPOSE_CONTROL	27

GENERAL_PURPOSE_DATA	27

SNOOZE	27

6.2.3 NET.CFG Example	28

6.3  Loading Multiple 21X4 Adapter Cards	28

6.4 Driver Error Messages	29

6.5  DOS ODI Driver Counters	51

6.5.1  General DOS ODI Driver Counters	51

MTotalTxPackets	51

MTotalRxPackets	51

MNoECBsAvailable	51

MTxTooBig	51

MTxTooSmall	51

MRxOverflow	51

MRxTooBig	52

MRxTooSmall	52

MTxMiscError	52

MRxMiscError	52

MTxRetryCount	52

MRxChecksumError	52

MRxMismatchError	52

MQueueDepth	52

6.5.2  Custom DOS ODI Driver Counters	53

Tx Watchdog	53

Tx Heart Beat Fail	53

Tx TP Link Fail	53

Tx Initially Deferred	53

Tx Single Collision	53

Tx Multiple Collisions	53

Tx FIFO Underflow	53

Tx Late Collision	54

Tx Loss or no Carrier	54

Tx Excessive Collisions	54

Rx Late Collision	54

Rx Watchdog	54

Rx Alignment Error	54

Rx FIFO Overflow	54

RX ECB Too Small	55

Num Restarts DC21X4	55

DMC Out Of Resources	55

Fatal System Error	55

Driver is halted	55

APPENDIX A:  Novell NetWare Environment.	55

Major Software Components	55

NetWare  Operating System	55

Workstation Software	56

NetWare Shell	56

Open Data Link Interface (ODI)	57

Multiple Link Interface Driver (MLIDS)	57

Link Support Layer (LSL)	58

Protocol Stacks	58

Drive Mappings	58

DOS Drive Pointers	59

NetWare Drive Pointers	59

Drive Pointer Types	59

Network Drive Assignments	59

Search Drive Assignments	60

Drive Pointer Assignments (MAP)	60

APPENDIX B: Hotline and Support Information.	60

Digital Semiconductor Information Line	60

Ordering Digital Semiconductor Products	60

Ordering Associated Literature	61

Associated Ethernet Controller Literature	61

Ordering Third-Party Literature	62

Electronic Orders	63

Telephone and Direct Mail Orders	63

Novell	64

- 1.	Overview

## 1.1	The DC21X4 Family of Products

The Intel 21X4 family consists of a series of Ethernet controllers sharing a generic architecture and, a set of support software, mainly drivers, each supporting the whole family of hardware components that existed when the software was released.

Below is the list of the current hardware and software components.

### 1.1.1	DC21X4 Hardware

The Digital Semiconductor hardware consists of the following Ethernet controllers:

- Intel 21040      PCI based 10                Mbps Ethernet Controller

- Intel 21041      PCI based 10                Mbps Ethernet Controller

- Intel 21140      PCI based 10/100 Mbps Ethernet Controller

- Intel 21140A PCI based 10/100 Mbps Ethernet Controller

- Intel 21142      PCI based 10/100 Mbps Ethernet Controller

- Intel 21143      PCI based 10/100 Mbps Family of Advanced Power managed Ethernet Controllers.

- Intel 21143      CardBus based 10/100 Mbps Ethernet Controllers.

- Intel 21540      PCI based 10/100 Mbps Ethernet Controller and Modem Interface

### 1.1.2	DC21X4 Software

The Intel 21X4 Family software provides support for the following environments:

- 32bit NDIS 21X4 drivers for Windows NT4.0, Windows NT3.51, Windows 95

Windows 95-OSR2 and Windows 98 (beta).

- NDIS2 21X4 MAC Driver for DOS, WfW3.11, OS/2 and Windows 95.

- Novell NetWare ODI-32 bit 21X4 Server Driver.

- Novell NetWare 3.1x, 4.x    ODI-16 bit DOS 21X4 Client Driver.

- SCO UNIX LLI D21X Driver.

- SCO UNIX MDI D21X Driver.

Note:    	Intel 21X4 Family software is backwards compatible all the way to the 21040 architecture, meaning that the latest software revision may be used on earlier hardware products as well.

- 2.     Purpose

This file explains how to install the Novell NetWare DOS ODI 16 bit Client driver Version 3.00.

This driver currently supports the:

- Intel 21040      PCI based 10                Mbps Ethernet Controller

- Intel 21041      PCI based 10                Mbps Ethernet Controller

- Intel 21140      PCI based 10/100 Mbps Ethernet Controller

- Intel 21140A PCI based 10/100 Mbps Ethernet Controller

- Intel 21142      PCI based 10/100 Mbps Ethernet Controller

- Intel 21143      PCI based 10/100 Mbps Ethernet Controller

- Intel 21143      CardBus based 10/100 Mbps Ethernet Controller

- Intel 21540      PCI based 10/100 Mbps Ethernet Controller and Modem Interface

- 3.      Supplied Files

Table 1. The    Novell NetWare ODI Client Directory Files

| File Name | Additional Keyword Details |
| --- | --- |
| README.DOC | This File. |
| RELEASE.TXT | Provides a description of updates to the new driver. |
| DC21X4.COM | DC21X4 Novell NetWare ODI 16bit Client driver. This file supports the Intel 2104x/2114x 10/100 Mbps Ethernet Controller based PCI/CardBus boards. |
| DC21X4.INS | Text file for NetWare* 4.x install |
| NET.CFG | Novell network configuration sample file for this driver. |

- 4.      Preinstallation Hardware and Software Requirements

## 4.1 Hardware Requirements

- An IBM-compatible personal computer with 80386 or higher	      microprocessor and a PCI bus and/or CardBus.

- A PCI or CardBus board based on Intel 2104x/2114x 10/100 Mbps based Ethernet controller.

## 4.2 Firmware Requirements

- Fully compliant Rev. 2.0 or above PCI BIOS supplied with the PC.

- If Ethernet adapter makes use of Intel 21050 (PCI-PCI bridge), PCI BIOS should support PCI-PCI bridge architecture.

- For CardBus adapter, Card and Socket Services software package.

## 4.3 Software Revision Requirements

- MS-DOS                        		version 3.31 or above.

If you are running EMM386, make sure you are using v4.49 or later. It is the version that ships with DOS 6.22.

- Novell LSL.COM        		version 2.16 or above.

- Novell IPXODI.COM	version 3.02 or above.

- Novell NETX.COM      	version 1.11 or above.

Or

Novell VLM files        	version 1.20B or above.

For getting the most updated revisions of Novell files, please

contact Novell at:

Novell, Inc.

Novell Labs Administration

MS E-13-1

122 East 1700 South

Provo, UT 84606

Tel: 801-429-7000

Internet: http://www.Novell.com/

- 5.	Installing 21X4 Based Hardware Boards

Installing a Intel 21X4 based PCI adapter in a PCI slot:

- While power is off, insert the adapter in one of your computer’s PCI slots

(please make sure the slot you select supports bus master operations!).

Motherboard with automatic PCI configuration (most PCs)

No specific setup is needed.

Motherboards with bus master jumpers and interrupt jumpers for PCI slots

- Enable bus master operations to the selected PCI slot using the appropriate motherboard jumper.

- Select interrupt to the selected PCI slot using the appropriate motherboard jumper.

- Start the computer.

In most cases, the PCI computer will automatically configure the adapter, but it may        happen that your PC BIOS has to be set up, in this case enter the setup menu of your machine during initialization (usually by pressing F1, F2, or Ctrl-Alt-S). Once in setup, find the entry for PCI slots and set these parameters (if they appear):

Slot Number:          		The slot where the adapter is installed (1-3)

Device:                			ENABLED

Master:                			ENABLED

Slave:                  			ENABLED

Latency Timer: 		DEFAULT (or at least 40 )

Interrupt:          			Choose an IRQ (usually from a predefined list).

Interrupt Edge-level:        	Level

I/O address:          		Choose an I/O address (usually from a predefined

list).

The exact wording of the parameters varies with different computers.

For CardBus, Insert a Intel 21143 based CardBus adapter, in a CardBus socket (no need to turn off the computer).

- 6.	Novell NetWare DOS ODI 16Bit Client Driver

## 6.1	Driver Installation

DC21X4.COM is a Novell NetWare ODI client driver supporting the Intel 21X4 family of PCI/CardBus based Ethernet controllers.

NOTE: For CardBus adapter make sure that the adapter inserted before loading the

driver.

### 6.1.1	Driver Installation for NetWare 3.11/3.12 clients

In order to install the driver from the DOS Command Line Prompt follow these steps:

- Insert a diskette containing the DC21X4 DOS ODI driver into the floppy drive.

- If first time installation then Create a subdirectory in your client's hard drive for storing the Novell NetWare DOS ODI client files.

- Copy the following files to the Novell Client subdirectory:

• LSL.COM		(Link Support Layer)

• DC21X4.COM		(Intel NetWare DOS ODI Client Driver)

• IPXODI.COM		(An IPX protocol stack file)

• NETX.EXE		(NetWare Shell, used with conventional memory workstations)

• NET.CFG		(Optional configuration file used for setting options different from 				the default settings).

The, LSL.COM, IPXODI.COM and NETX.EXE files can be obtained 	from the Workstation Driver Diskette in the Novell Client Software Package.

- Edit the NET.CFG file according to your needs:

Select the Frame type needed (if other than default), if you do not know which frame type to load, ask your system administrator or type CONFIG at your file      server console to see which frame types your file server is using, then select one of these frame types on the workstation (NetWare default frame type is 802.3).

If you list two or more NetWare frame types in the NET.CFG, the first frame

type will bind to the IPX protocol.

Note

- NO need to specify    SLOT NUMBER, BUS ID or INTERRUPT NUMBER!

- If you specify Bus ID please notice that for PCI bus you write

BUS ID    PCI 4

And for CardBus you write :

BUS ID PCMCIA 3

in NET.CFG

- If you specify Slot Number please notice that SLOT 1 means    in PCI : The first board found in system.

- The NET.CFG file should be in the same directory as the adapter driver.

- For CardBus adapter, Card and Socket Services must be loaded prior to the Novell Netware client driver loading sequence.

- Reserve Logical Volumes for NetWare

Edit your CONFIG.SYS and make sure that the LASTDRIVE entry (if exists) is not “Z”.

Usually it will be one letter pass the last volume , for instance if you have a Hard disk (C)    and an compact disk (D)    then please write

LASTDRIVE=E

- Load the Novell NetWare client files

- To manually load these files from the DOS prompt, type the following commands in the given order to initialize DOS ODI driver support and the IPX protocol stack:

LSL 		Followed by <Enter>

DC21X4 		Followed by <Enter>

IPXODI 		Followed by <Enter>

NETX 			Followed by <Enter>

- To automatically load these files, add these lines to your C:\AUTOEXEC.BAT file (and in the following order):

<subdirectory>LSL

<subdirectory>DC21X4

<subdirectory>IPXODI

<subdirectory>NETX

Save the file and reboot the workstation. After the commands execute and if a server is present, the workstation will attach to the server and display the server’s name.

- Log onto the network.

The installation of the NetWare DOS ODI client driver is now complete.

### 6.1.2	Driver Installation for NetWare 4.1/3.12 clients with VLMs

The NetWare DOS Requester (VLMs) is the requester component of the NetWare

16-bit Client for DOS and MS Windows. VLM is the primary technology that

provides access to NetWare 4.x servers from the DOS environment.

For a first time installation of a VLM NetWare client please use the Novell INSTALL.EXE utility.

INSTALL.EXE is located on NetWare    "Workstation for DOS" disk.

NOTE: INSTALL.EXE must be version 1.21 or greater

#### 6.1.2.1	First Installation of NetWare 4.1/3.12 clients with VLMs

Run the INSTALL.EXE program, NetWare Client Install program screen appears. Fill in the information requested by the program. You must complete each of the options as they are specific to your needs.

Step #5 asks you to select a network driver and provide configuration options. It uses the specifications you select to create    the NET.CFG file. You may select the Intel DC21X4 driver from the list Novell    provides, or if you have a more updated driver choose    OTHER DRIVER and insert the Intel Drivers diskette.    Select the Intel 2104x/2114x 10/100 Mbps Ethernet Controller Driver. There are several configurable settings, we will list the most common ones:

- Node Address: (optional) Changes the Node Address to a local address.

- Frame Type: 	Set the frame type to match the file server (NetWare default is the 802.2 frame type).

- Connection Type: (Optional) The type of connection to work with (Autosense is recommended)

- Line Speed: (Optional) Sets the Line speed to either 10 or 100 Mbps.

Meaningful only when TP or TP_FULL_DUPLEX are selected (See Driver Keywords section    for details)

In the next step the install program will copy the appropriate files to your hard drive. This is the final step.

Note: Be sure to add LASTDRIVE=Z to your CONFIG.SYS file

and, FIRST NETWORK DRIVE=<First available Drive Letter>    in the NET.CFG file under the NetWare DOS Requester section.

Usually the <First available Drive Letter> is F

#### 6.1.2.2	Driver Upgrade of NetWare 4.1/3.12 clients with VLMs

To manually install the DC21X4.COM driver or to upgrade the driver

- Insert a diskette containing the DC21X4 DOS ODI driver diskette into the floppy drive.

- If first time installation then Create a subdirectory in your client's hard drive for storing the Novell NetWare DOS ODI client files and Copy the Novell NetWare files to the Novell Client subdirectory:

- Copy the DC21X4.COM driver file to the Novell Client subdirectory.

- Edit the NET.CFG file according to your needs.

Note: Be sure to add , FIRST NETWORK DRIVE=<First available Drive Letter> in the NET.CFG file

under the NetWare DOS Requester section.

Usually the <First available Drive Letter> is F

- For CardBus adapter, Card and Socket Services must be loaded prior to the Novell

Netware client driver loading sequence.

- Load the Novell NetWare client files

- To manually load these files from the DOS prompt, type the following commands in the given order to initialize DOS ODI driver support and the IPX protocol stack:

LSL		Followed by <Enter>

DC21X4 		Followed by <Enter>

IPXODI 		Followed by <Enter>

VLM			Followed by <Enter>

- To automatically load these files, add these lines to your C:\AUTOEXEC.BAT file (and in the following order):

<subdirectory>LSL

<subdirectory>DC21X4

<subdirectory>IPXODI

<subdirectory>VLM

Save the file and reboot the workstation. After the commands execute and if a server is present, the workstation will attach to the server and display the server’s name.

Note:    Be sure to add LASTDRIVE=Z to your CONFIG.SYS file.

The DC21X4.COM may be loaded high in order so free DOS conventional memory for other

use (this is done by writing  LH <subdirectory>DC21X4 in autoexec.bat)

- Log onto the network.

The installation of the NetWare DOS ODI client driver is now complete.

## 6.2	Driver Keywords

Novell NetWare DOS Client    environment uses a configuration file called NET.CFG in order to control the various NetWare components of the system. This file usually resides in the Novell NetWare subdirectory.

For each workstation driver being configured, if section headings and options deviate from the established defaults of the ODI software or, if multiple protocols are being used, then select a set of supported values from the DC21X4 configuration options. These values should be written in the NET.CFG file and then load the driver.

To change any default settings after the driver is loaded, the user must modify the current NET.CFG file (or create such a file if it does not exist), using any text editor, unload the Novell protocol files and driver, and reload them again.

To unload the Novell protocol do the following:

VLM U (or NETX U)	Followed by <Enter>

IPXODI U 		Followed by <Enter>

DC21X4 U 		Followed by <Enter>

LSL U		Followed by <Enter>

Note: -	For CardBus the adapter must be removed from the socket prior to the unloading sequence ,before reloading the driver the adapter must be reinserted.

-	See the Novell NetWare ODI DOS installation documentation for additional information.

### 6.2.1	NET.CFG Conventions

The NET.CFG is a text file divided into sections shown in the following example:

Link Support

Max Boards 4

Max Stacks 8

Protocol	        <Protocol Name>

Bind    <Driver Name>

Link Driver        <Driver Name>

Frame        ETHERNET_802.3

Frame        ETHERNET_II

Frame        Ethernet_802.2

Frame        Ethernet_SNAP

NetWare DOS Requester

FIRST NETWORK DRIVE = F

NETWARE PROTOCOL = NDS BIND

Note: 	Where <Driver Name> above is DC21X4 and <Protocol Name> is usually IPX.

The NET.CFG file structure follows these guidelines:

- Main section headings must be left-justified and are not case sensitive.

- Options are not case sensitive and must be preceded by a tab or spaces.

- Options must follow the heading associated with them.

- Comments are preceded by a semicolon (“;”) and, end each line with an <Enter>.

- Numbers are in decimal notation unless noted otherwise.

### 6.2.2	DC21X4    Keywords

Below is a list of    the driver keywords (options) defined for the DC21X4.COM DOS ODI Link Driver. The keywords were divided    into two main groups; one is the Novell NetWare Operating System Keywords and the other is the DC21X4 specific keywords.

#### Novell NetWare Operating System Keywords

Novell NetWare provides standard driver keywords, The most common ones are listed here.

The keywords are written in the following format:

Keyword Name

Short explanation

Format:                                  The format of the keyword as written in NET.CFG

Range:                                    The possible range of values for this keyword.

Supported values: The sub-set of values (within range) supported by the 21X4 				                                  driver or device.

Default Value:                The value the driver will use if this keyword will not be written.

NOTE: Some of the standard keywords have no meaning for the 21X4 driver or should not be used in PnP systems (which PCI and CardBus systems are part of); The driver will ignore these keywords if found in NET.CFG.

These Keywords will also be presented here for clarity.

##### DMA

Selects the DMA channel to be used by the adapter.

This keyword has no meaning for DC2X4 devices since they are Bus-Master DMA devices.

Format: 	DMA    <Hexadecimal Value>

Range:	N.A

Supported values: 	N.A

Default Value:	N.A

Note: The driver Ignores this Keyword.

##### IRQ

Indicates to the driver which Interrupt channel the device is connected to.

In PCI the BIOS is responsible for assigning this value, the driver always uses the BIOS settings and ignores this keyword.

Format: 	IRQ    <decimal Value>

( You May also write:    INT <decimal Value>).

Range:	0 - 15

Supported values: 	0 - 15

Default Value:	According to the PCI BIOS settings.

Note: The driver Ignores this Keyword.

##### MEM

The Physical memory address of the 21X4 registers.

In PCI the BIOS is responsible for assigning this value.

The 21X4 device registers may be accessed via the memory address space (memory mapped registers) or via the I/O address space (I/O mapping). The DC21X4.COM driver always accesses the 21X4 registers via the I/O address space.

This keyword is ignored by the driver.

Format: 	MEM    <Hexadecimal Value>

Range:	0 - FFFFFFFF

Supported values: 	N.A

Default Value:	N.A

Note: The driver Ignores this Keyword.

##### PORT

The I/O base address of the 21X4 based adapter card.

In PCI the BIOS is responsible for assigning this value and the driver will use the BIOS settings and ignore this keyword.

Format: 	PORT    <Hexadecimal Value>

Range:	0 - FFFF

Supported values: 	0 - FFFF

Default Value:	According to the PCI BIOS settings.

Note: The driver Ignores this Keyword.

##### BUS ID

Tells the driver software to scan the specified bus for the presence of the 21X4 based adapter card. If this keyword is not specified the driver will scan all the supported buses for the device.

Format: 	BUS ID    <Bus Name> <Bus Value>

Range:			ISA		00h

MCA		01h

EISA		02h

PCMCIA	03h

PCI		04h

VESA		05h

BUS_ID_HSM_DEFAULT	0FFh

Supported values: 	PCI		04h

PCMCIA	03h

BUS_ID_HSM_DEFAULT	0FFh

Default Value:	BUS_ID_HSM_DEFAULT (Search on all supported busses)

##### SLOT

Tells the driver software to search for the device in the specified slot.

If this keyword is not specified the driver will search for the first device occurrence.

Format: 	SLOT    <decimal Value>

Range:	1 - 64,000

Supported values:      1 - 255

Default Value:	Scan for first device occurrence

Note: In PCI, Slot number means Device occurrence and not Physical slot Number as in EISA bus.

##### NODE ADDRESS

Overrides the node address read from the network board. The node address is a 12-digit hexadecimal number.

Format: 	NODE ADDRESS      < 12 digit Hexadecimal Value>

Range:	See description above.

Supported values:      All in Range.

Default Value:	The Node Address as read From the Nic.

Note:        Node Address values should follow these conventions:

- The first bit in the second digit must always be 0. For example, the following addresses are INVALID: 01 00 00 00 00 00 (Hex) to    01 FF FF FF FF FF    (Hex).

- The second bit in the second digit should be 1. If it is 0, the driver will modify it to 1, Hence

A node address value of    000000000012(hex) will be modified to 020000000012(hex).

##### FRAME

Selects the NetWare Ethernet frame types to be used by the driver.

Format:          FRAME < Frame Type>

Range:	Ethernet_802.2

Ethernet_802.3

Ethernet_II

Ethernet_SNAP

Supported values:      All in Range.

Default Value:	Ethernet_802.2 (NetWare 3.12, 4.x)

Ethernet_802.3 (NetWare 3.11)

Note: See the IEEE Ethernet standard and Novell documentation for further details.

##### PROTOCOL

Indicates the protocol types to be used and the Frame type each protocol will be using.

Each Frame type has corresponding Protocol ID numbers for IPX and other protocols, This number must also be written in the Protocol entry.

Format:          PROTOCOL < Protocol Type><Protocol ID><Frame Type>

Range:            	See    Novell NetWare documentation.

Supported values:      All in Range.

Default Value:	Ethernet_802.2 (NetWare 3.12, 4.x)

Ethernet_802.3 (NetWare 3.11)

Note: See the IEEE Ethernet standard and Novell documentation for further details.

Examples:

PROTOCOL IPX          0                ETHERNET_802.3

PROTOCOL IPX          8137    ETHERNET_II

PROTOCOL IPX          E0            ETHERNET_802.2

PROTOCOL IPX          8137    ETHERNET_SNAP

PROTOCOL IP              0800    ETHERNET_II

PROTOCOL ARP      0806    ETHERNET_II

PROTOCOL RARP 8035    ETHERNET_II

#### Driver Specific Keywords

DC21X4 driver specific keywords are listed below; For each keyword the following information will be provided:

Keyword Name

Short explanation

Format:                                  The format of the keyword as written in NET.CFG

Range:                                    The possible range of values for this keyword.

Default Value:              The value the driver will use if this keyword will not be written.

These keywords can be divided into several groups:

- Transmit and Receive control.

- PCI settings.

- Connection/Media Selection.

Driver selected connection is a combination of Line_Speed    selection and Media type selection.

The following two tables describe how does the driver determine the actual media connection from these two parameters.

First the driver determines the Line_Speed according to the following table:

| Line Speed Keyword Value | Intel Device Number | Driver Line Speed selection (in Mbps) |
| --- | --- | --- |
| None | 21040, 21041 | 10 |
| None | 21140, 21142,21143 and Above | 100 |
| 10 | All 21X4 devices | 10 |
| 100 | All 21X4 devices | 100 |

Then the driver determines the connection type according to the following table:

| Media Keyword Value | Driver Selected Line Speed (in Mbps)* | Driver Selected Connection |
| --- | --- | --- |
| None | 10, 100 | Read from Nic |
| AUTOSENSE | 10, 100 | AUTOSENSE |
| AUI | 10, 100 | AUI (10Base5) |
| BNC | 10, 100 | BNC (10Base2) |
| T4 | 10 | TP (10BaseT) |
| T4 | 100 | 100BaseT4 |
| TP | 10 | TP (10BaseT) |
| TP | 100 | 100BaseTX |
| TP_FULL_DUPLEX | 10 | TP Full-Duplex (10BaseT-FD) |
| TP_FULL_DUPLEX | 100 | 100BaseTX-FD |
| TP_NLT | 10 | 10BaseT No-Link-Test |
| TP_NLT | 100 | 100BaseTX |

Note: If more than one media KEYWORD is written in NET.CFG then the last entry will be used.

Note: *See previous table.

Media presence in a Nic is implementation dependent but there are media types which can not possibly be used with some of the 21X4 devices; as an example

the 21040 devices are 10Mbps devices only and it is not possible to use them in a 100Mbps network.

The following table summarizes the possible Device-Media combinations:

| 21X4 Device Type | TP | TP FD | BNC | AUI | 100 Base TX | 100 Base TX FD | 100 Base T4 | Power-up Autosense | Dynamic (run-time) Autosense |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21040 | Ö | Ö | Ö | Ö | - | - | - | Ö | - |
| 21041 | Ö | Ö | Ö | Ö | - | - | - | Ö | Ö |
| 21140 | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö | - |
| 21140A | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö |
| 21142 | Ö | Ö | Ö | Ö | - | - | - | Ö | Ö |
| 21143 | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö |
| 21540 | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö | Ö |

Note: The 21142 device supports the 100Mbps media via an MII PHY only.

##### Transmit Threshold (TX_THRESHOLD)

Controls the minimum amount of packet's data	(in bytes) the 21X4 transmit FIFO must contain before it will start transmitting the packet.

Format: 		TX_THRESHOLD <Hexadecimal Value>

Range:    		0 - 4

Default Value:	1          In 21040, 21041 and CardBus devices.

0          In all other devices.

| TX_THRESHOLD Keyword Value | Actual 21x4 Threshold (10Mbps) | Actual 21x4 Threshold (100Mbps) |
| --- | --- | --- |
| 0 | 72 | 128 |
| 1 | 96 | 256 |
| 2 | 128 | 512 |
| 3 | 169 | 1024 |
| 4 | Full Packet | Full Packet |

Note: TX Threshold value of 4 will be interpreted in 21040 and 21041 devices as 169 bytes threshold

Note:                  Changing this parameter may affect the device performance.

##### Number of Transmit Buffers (TX_BUFFERS)

Selects the number of transmit buffers the driver will work with.

Format: 		TX_BUFFERS <Hexadecimal Value>

Range:		2 - 3.

Default value : 	3

##### Number of Receive Buffers (RX_BUFFERS)

Selects the number of receive buffers the driver will allocate and work with.

A buffer is 1568 bytes long.    Adding buffers will increase the driver size when resident in memory but will usually improve performance.

Format: 		RX_BUFFERS <Hexadecimal Value>

Range:		2 - 10 hex (16 Decimal).

Default value:    	10 (Hex)

##### ERI

Enables the Chip's Early Receive function.

Early receives allows a frame to start being processed before it is completely transferred    to the system memory, a process which usually increases performance.

Format: 		ERI

Range:		None

Default value:    	ERI disabled

Note:      1. ERI is NOT supported in 21040 and    21140 devices..

2. ERI should be disabled when loading    MS-WFW3.11 and using its

TCPIP protocol stack over the ODI driver.

3. Changing this parameter may affect the device performance.

##### Configuration Latency Timer (CFLT)

Specifies, in units of PCI bus clocks, the value	of the latency timer.

This is a PCI related parameter.

Format: 		CFLT <Hexadecimal Value>

Range:    		0 - FF Hex

Default value : 	According to PCI BIOS settings.12 (hex)

Note:    Changing this parameter may affect the device performance.

If CFLT value is less than 12(hex) then the driver will modify it to 12(hex)

##### Programmable Burst Length (PBL)

Changes the Maximum number of longwords to be transferred in one DMA transaction.

This is a PCI related parameter.

Format: 		PBL <Hexadecimal Value>

Range:    		0, 2, 4, 8, 10 or 20 Hex

(0 means Unlimited burst, 20Hex means 32 Longwords            burst).

Default value : 	10h (16 longwords) in 21040 and 21041 devices

20h (32 Longwords) in 2114x and 2115x Devices.

Note:      1.    Changing this parameter may affect the device performance.

2.    PBL may be limited by the driver due to system limitations.

3. If altering PBL It is recommended to program both PBL and CAL to the same burst    value.

##### Cache Alignment    (CAL)

Controls the Programmable address boundaries for the data burst stop.

This is a PCI related parameter.

Format: 	CAL < Hexadecimal Value>

Range:    		1        (8 Longwords)

2        (16 longwords)

3        (32 Longwords)

Default value : 	2        (16 longwords) in 2104x devices

3        (32 Longwords) in 2114x and 2115x Devices.

Note:      1. Changing this parameter may affect the device performance.

2. CAL may be limited by the driver due to system limitations.

3. It is recommended to program both PBL and CAL to the same burst    value.

##### Memory Read Multiple (MRM)

Enables the Chip's Memory Read Multiple Extended PCI command.

This command is semantically identical to the Memory Read command except that it additionally indicates that the master may intend to fetch more than one                                                            cache line before disconnecting.

Format: 		EXT_MRM <Decimal Value>

Range: 		0	(Disabled)

1	( Enabled)

Default value : 	1 	(Enabled).

Note:      1. Supported in 21140A, 21142, 21143 and later devices

2. Disabling this command may affect the device performance.

##### Memory Read Line (MRL)

Enables the Chip's Memory Read Line PCI command.

This command is semantically identical to the Memory Read command except that it additionally indicates that the master may intend to fetch a complete cache line.

Format: 		EXT_MRL <Decimal Value>

Range: 		0 	(Disabled)

1 	(Enabled)

Default value : 	1 	(Enabled).

Note:      1. Supported in 21140A, 21142, 21143 and later devices

2. Disabling this command may affect the device performance.

3. Due to incompatibility problems with certain PCI host bridges and PCI-to-PCI bridges,

the driver currently does not enable the Memory Write Invalidate (MWI) and Memory Read Line (MRL) commands simultaneously. The driver will only enable one of the two commands; by default, MRL will be enabled.

##### Memory Write Invalidate (MWI)

Enables the Chip's Memory Write Invalidate PCI command.

This command is semantically identical to the Memory Write command except that it additionally guarantees a minimum transfer of one complete cache line; i.e., the master may intends to write all bytes within the addressed cache line in a single PCI transaction unless

interrupted by the target.

Format: 		EXT_MWI <Decimal Value>

Range: 		0 	(Disabled)

1 	(Enabled)

Default value : 	1	(Enabled).

Note:      1. Supported in 21140A, 21142, 21143 and later devices

2. Disabling this command may affect the device performance.

3. Due to incompatibility problems with certain PCI host bridges and PCI-to-PCI bridges,

the driver currently does not enable the Memory Write Invalidate (MWI) and Memory Read Line (MRL) commands simultaneously. The driver will only enable one of the two commands; by default, MRL will be enabled.

##### TURBO

Changes the behavior of the Host-to-PCI chipset, giving a higher priority to the PCI devices over other system devices. This was relevant only to a certain family of Host-to-PCI chipsets used in early PCI machines. In these machines

the increase rate of Transmit-Underflows and Receive-Overflows counters directly reflect a low PCI bus priority and low device performance.

Format: 		TURBO

Range: 		None

Default value : 	None

Note:    The use of this keyword alters the system behavior and is not recommended.

##### TP

Selects the 10BaseT Media

Format: 		TP

Range: 		None

Default value : 	None

Note:    The actual connection (10BaseT or 100BaseTX) is done in conjunction with the Line_Speed keyword.    See the beginning of this section for further media selection details.

##### TP FULL DUPLEX

Selects the 10BaseT Full-Duplex    Media

Format: 		TP_FULL_DUPLEX

Range: 		None

Default value : 	None

Note:    The actual connection is done in conjunction with the Line_Speed KEYWORD.    See the beginning of this section for further media selection details.

##### TP NO LINK TEST

Selects the 10BaseT media but disables the link check operation done, by the 21X4 device, when    10BaseT medium is selected.

Format: 		TP_NO_LINK_TEST

Range: 		None

Default value : 	None

Note:    The actual connection is done in conjunction with the Line_Speed keyword.    See the beginning of this section for further media selection details.

This keyword is    used when connecting the 21X4 board to old 10BaseT HUBs which do not transmit Link Pulses.

##### BNC

Selects the 10Base5 (BNC)    media

Format: 		BNC

Range: 		None

Default value : 	None

##### AUI

Selects the 10Base2 (AUI) media

Format: 		AUI

Range: 		None

Default value : 	None

##### T4

Selects the 100BaseT4 Media

Format: 		T4

Range: 		None

Default value : 	None

Note:    The actual connection (10BaseT or 100BaseT4) is done in conjunction with the Line_Speed keyword.    See the beginning of this section for further media selection details.

##### LINE_SPEED

Selects the line speed (10Mbps or 100Mbps).

Ignored for 21040 and 21041 devices.

Format: 		LINE_SPEED < Decimal value>

Range: 	 	10, 100

Default value : 	Chip dependent.

Note:    The actual connection    is done in conjunction with one of the media    keywords.    See the beginning of this section for further media selection details.

##### AUTOSENSE

Enables automatic sensing of the medium connected to the network (autosense) among the NIC's media when the driver loads (Also called Power-Up Autosense), and at run-time (also called Dynamic Autosense).

The autosense algorithm is highly dependent on the 21X4 device type being used and board implementation.

Format: 		AUTOSENSE

Range: 	 	None

Default value : 	None

Note: The following list summarizes the 21X4 devices autosense capabilities:

- DC21040 device

Power-Up Auto-Sensing only between TP, BNC and AUI media.

- DC21041 device

Power-Up and run-time auto-sensing    between TP, BNC and AUI media.

- DC21140, DC21140A devices

If MII Phy is present :

Power-up and runtime autosense using the MII Phy media only.

Otherwise

Power-Up and runtime autosense between all media capable of notifying their link status via the 21X4 General Purpose Pins    (board implementation dependent).

- DC21142 device

If MII Phy is present :

Power-up and runtime autosense using the MII Phy media only.

Otherwise

Power-Up and runtime autosense between TP, BNC and AUI media.

- DC21143 and above devices

If MII Phy is present :

Power-up and runtime autosense using the MII Phy media only.

Otherwise

Power-Up and runtime Autonegotiation    between all media except

when both AUI and BNC are present, in this case BNC will not be sensed.

##### MULTIA

Notifies the drivers that it is loaded on a Digital Equipment Corporation MULTIA PC.

Format: 		MULTIA

Range: 	 	None

Default value : 	None (standard PC)

##### GENERAL_PURPOSE_CONTROL

Overrides the control value in the GENERAL_PURPOSE register on network cards based on DC21140 and above devices. 	Normally, this value should not be changed.

Format: 		GENERAL_PURPOSE_CONTROL <Hexadecimal Value>

Range: 		0 - FFFF

Default value : 	Read from Nic.

Note: This keyword is for engineering purpose only, this value should not be changed.

##### GENERAL_PURPOSE_DATA

Overrides the data value written to the GENERAL_PURPOSE register on network cards based on DC21140 and above devices.

Format: 		GENERAL_PURPOSE_DATA <Hexadecimal Value>

Range: 		0 - FFFF

Default value : 	Read from Nic.

Note: This keyword is for engineering purpose only, this value should not be changed.

##### SNOOZE

Changes the 21X4 devices power-management mode to Power-Saving mode.

Format: 		SNOOZ

Range: 		None

Default value : 	Fully powered.

Note: The 21040 device does not support snooze mode.

### 6.2.3	NET.CFG Example

;--------------------------------------------------------------------------------------------------

; Network Configuration File for NOVELL DC21X4 DOS ODI Client driver.    ;

; supports 21X4 based Ethernet boards on Novell NetWare 3.12 or 4.x.            ;

;--------------------------------------------------------------------------------------------------

Link Support

Max Boards 4

Max Stacks 8

Protocol    IPX

Bind DC21X4

Link Driver DC21X4

Frame        Ethernet _802.3

Frame        Ethernet _II

Frame        Ethernet_802.2

Frame        Ethernet_SNAP

Protocol 	IPX          0              	Ethernet _802.3

Protocol 	IPX          8137	Ethernet _II

Protocol 	IPX          E0          	Ethernet_802.2

Protocol 	IPX          8137    	Ethernet_snap

Protocol 	IP              0800 	Ethernet _II

Protocol 	ARP      0806    	Ethernet _II

Protocol 	RARP 8035    	Ethernet _II

; Optional DC21X4 driver keywords:

;-----------------------------------------------

AUTOSENSE

NetWare DOS Requester

FIRST NETWORK DRIVE = F

NETWARE PROTOCOL = NDS BIND

Note:	Comment lines are preceded by a semi-colon (“;”).

## 6.3 	Loading Multiple 21X4 Adapter Cards

In order to load multiple adapters please follow these steps:

- Create LINK DRIVER entry in NET.CFG for each adapter installed.

- Each entry must have a SLOT    keyword having a unique    slot number .

- Each entry must have its own FRAME type.

- The driver must be loaded again for each new 21X4 based adapter board installed in the system.

NET.CFG (for multiple NICs)

LINK SUPPORT

Max Boards 4

Max Stacks 8

PROTOCOL    IPX

Bind 1

LINK DRIVER DC21X4

FRAME Ethernet_802.3

SLOT 1

LINK DRIVER DC21X4

FRAME Ethernet _802.2

SLOT 2

NETWARE DOS REQUESTER

FIRST NETWORK DRIVE = F

## 6.4	Driver Error Messages

Message	DC21X4-DOS-1: The LSL is not loaded.

Reason	The Link Support Module program (LSL.COM) is not loaded.

Action	Load the LSL.COM before loading the driver.

Message	DC21X4-DOS-2:  The LSL has no more room for a board 			using Frame <Frame Type>.

Reason	The LSL has no free entry for registering another board,    whether virtual

or physical. The DOS ODI LSL can support up to eight boards.

Action	Reduce the number of active boards in the system by unloading a driver

or by decreasing the number of frame types activated by the drivers written in NET.CFG .

Message	DC21X4-DOS-3:  Could not find DC21X4 driver to unload

Reason	A request was made to unload the DC21X4 driver, but the driver is not

loaded.

Action	None

Message	DC21X4-DOS-4:  A TSR is loaded above the DC21X4 driver.

Reason	A request was made to unload the DC21X4 driver, but the driver detected

a Terminate-and Stay-Resident (TSR) program loaded above it and hence cannot unload itself.

Action	Unload the TSR loaded above the driver and then try this operation again.

Message	DC21X4-DOS-6:  The adapter did not initialize.  				DC21X4 did not load.

Reason	The DC21X4 hardware or software initialization failed. The DC21X4 driver

did not load.

Action	Usually this message is preceded by a more specific error message.

Please handle that error and try to load the driver again.

Message	DC21X4-DOS-7:  You need another DC21X4 driver Section 			Heading in the NET.CFG file in order the load the 			LAN driver again.

Reason	You have    tried to load the DC21X4 driver more than once, usually this is

done in order to activate another 21X4 based board installed in the

system.    This requires a LINK DRIVER section for each driver being

loaded. The driver did not find its associated entry in NET.CFG and failed

to load.

Action	Edit the NET.CFG file and add a LINK DRIVER entry for each

driver instance you will be loading. Remember to add the SLOT keyword

in each entry in order to associate the driver with the appropriate board.

Then unload the drivers and reload them again.

Message	DC21X4-DOS-8: A NET.CFG is required to load the driver

again.

Reason	You have    tried to load the DC21X4 driver more than once, usually this is

done in order to activate another 21X4 based board installed in the

system.    This requires a LINK DRIVER section for each driver being

loaded. The driver did not find its associated NET.CFG file at all causing it to fail loading.

Action	Create a NET.CFG file and, add a LINK DRIVER entry for each driver

instance you will be loading. Remember to add the SLOT keyword

in each entry in order to associate the driver with the appropriate board.

Then unload the drivers and reload them again.

Message	DC21X4-DOS-9: The following configuration file entry

was ignored:

<Entry Name>

Reason	The DC21X4 driver ignored the <Entry Name> NET.CFG entry due to a

keyword parsing error.

Action	Please modify the appropriate keyword entry in NET.CFG file to contain

valid parameters, or delete the entry in question.

Message	The DC21X4 driver has been successfully removed.

Reason	A request was made to unload an DC21X4 driver, and the driver was

successfully removed from memory.

Action	None

Message	DC21X4-DOS-11: The driver does not support frame

<Frame Type> The protocol keyword has been ignored.

Meaning	The "PROTOCOL" option that was specified in the NET.CFG for the

DC21X4 driver contains a frame type that is not supported by the driver.

Action	Check the "PROTOCOL" line in the NET.CFG file for spelling errors.

Example of valid PROTOCOL entries is given below:

Protocol 	IPX          0              	Ethernet _802.3

Protocol 	IPX          8137	Ethernet _II

Protocol 	IPX          E0          	Ethernet_802.2

Protocol 	IPX          8137    	Ethernet_snap

Protocol 	IP              0800 	Ethernet _II

Protocol 	ARP      0806    	Ethernet _II

Protocol 	RARP 8035    	Ethernet _II

Message	DC21X4-DOS-12:  The protocol keyword must have a 				frame type.  Entry ignored.

Reason	The "PROTOCOL" option was specified in the NET.CFG for a DC21X4

driver but is missing the associated frame type.

Action	Add the frame type to the "PROTOCOL" option.

Example of valid PROTOCOL entries is given below:

Protocol 	IPX          0              	Ethernet _802.3

Protocol 	IPX          8137	Ethernet _II

Protocol 	IPX          E0          	Ethernet_802.2

Protocol 	IPX          8137    	Ethernet_snap

Protocol 	IP              0800 	Ethernet _II

Protocol 	ARP      0806    	Ethernet _II

Protocol 	RARP 8035    	Ethernet _II

Message	DC21X4-DOS-13:  The driver could not register Protocol

ID <Protocol ID> for protocol stack <Protocol Name> for

frame type.

Reason	The DC21X4 driver failed to register the specified Protocol ID with the

LSL.

Action	Verify the protocol information in the NET.CFG file for spelling errors.

Message	DC21X4-DOS-15:  The frame type is already activated 			for frame <Frame Type>.

DC21X4-DOS-9: The following configuration file entry

was ignored:

<Frame Type>

Reason	Two    FRAME    keywords under the same LINK DRIVER section specified

the same frame type. A frame type may be specified only once per driver.

Action	Remove the duplicate "Frame" keyword entry.

Message	DC21X4-DOS-16:  The node address was incorrectly 				specified in NET.CFG.

DC21X4-DOS-9: The following configuration file entry

was ignored:

NODE ADDRESS <Address>

Reason	The "NODE ADDRESS" keyword in the NET.CFG file has too many digits.

The keyword value will be ignored and the driver will use the node address read from the 21X4 board.

Action	Specify a valid node address in NET.CFG (with 12 digits at most).

Message	DC21X4-DOS-17:  An invalid keyword was specified in 			NET.CFG on line <Line Number>.

Reason	The keyword specified in NET.CFG file is incorrect.

Action	Check the keyword in question.    Make sure that the keyword is correctly

spelled.

Message	DC21X4-DOS-18:  The frame type specified in NET.CFG 			is not supported.

Reason	The  FRAME keyword specified in the NET.CFG for the DC21X4

driver is not supported.

Action	Check the FRAME keyword data for spelling errors and the Novell

NetWare documentation for valid FRAME types.

Example of valid FRAME entries is given below:

Frame		Ethernet _802.3

Frame 		Ethernet _II

Frame    		Ethernet_802.2

Frame 		Ethernet_snap

Message	DC21X4-DOS-19:  An invalid Ethernet node address is 			specified in NET.CFG.

The driver modified the incorrect address bits.

Reason	The NET.CFG has an invalid node address keyword entry.    The DC21X4

driver modified and corrected the address.

An Ethernet address is twelve characters in length.

This error occurs

- When Bit 0 of the second address character is a 1.

This bit must always be 0.

or

- When Bit 1 of the second address character is a 0.

This bit must always be 1.

For example, a node address of 000000000012 will be modified to 020000000012.

A node address of 030000000012 will be modified to 020000000012;

and a node address of 010000000012 will also be modified to 020000000012.

Action	Please correct the NODE ADDRESS    value written in    NET.CFG file.

Message	DC21X4-DOS-27: This driver requires LSL version 2.11 or

higher.

Reason	The version of the LSL loaded in the system is outdated and can

not be used by the DC21X4 driver.

Action	Please upgrade your Novell NetWare client software.

Message	DC21X4-DOS-29:  The driver did not load.

Reason	The driver failed to load in the system for a reason usually given via an

Error message preceding this message.

Action	Please relate to the Error message preceding this message.

Message	DC21X4-DOS-31:  A missing or invalid value was

specified on line <Line Number>

Reason	The DC21X4 driver encountered an error while parsing its entry in

NET.CFG. The reason for that error is an Invalid keyword value or

that no value was found at all.

Action	Please modify line <Line Number> entry in NET.CFG file to contain

valid parameters or delete the entire line.

Message	FATAL: Failed to allocate memory

Reason	Driver call to OS memory support routines for dynamic memory

allocation has failed.

Action	1. Unload unneeded applications from memory and try loading the driver

again.

2. If previous action did not help, reboot the system.

Message	FATAL: Failed to lock system memory or retrieve its

physical address

Reason	Driver call to OS memory support routines for locking its internal buffers

and descriptors failed.

Action	Check the memory manager documentation and make sure that it

supports a memory manager VDS (Virtual DMA Services).    If in doubt,

either use another memory manager or do not use your current memory

manager.

If you're running EMM386, make sure you are using v4.49 or later. It is the version that ships with DOS 6.22.

Upgrade you DOS version to 6.22

Message	FATAL: DC21X4 driver works only with 386 or above!

Reason	The driver failed to identify the CPU as a 386 or above.

The    driver can run on 386 (or above) machines only.

Action	Please run the driver on 386 or above machines.

Message	DC21X4-DOS-50: FATAL: Board not found in system

Reason	The DC21X4 driver could not detect a 21X4 based board in the system.

Action	Install your Nic in the system as described in chapter 5 of this installation

guide.

If you are using an on-board device, check the PC BIOS for proper device

settings and verify the device is enabled.

If you are using a Nic, check the appropriate PC BIOS slot settings

and verify the device is enabled.

Verify adapter is installed correctly in the PCI slot.

Check your NET.CFG file for proper SLOT or BUS ID keywords values for

the device; If this is single device remove both SLOT and BUS ID

keywords all together.

Upgrade your DC21X4.COM driver to the latest version.

Message	DC21X4-DOS-200: FATAL: Timeout when initializing DC21X4

with setup packet!

Reason	The chip failed to read the setup packet within a predefined amount of

time. This will prevent the 21X4 device from working properly.

Action	Make sure the PC BIOS setup or motherboard switches are set to enable

DMA or Bus Master operation for this slot or on-board device.

Enter the setup menu of your machine during initialization (usually by pressing F1, F2, or Ctrl-Alt-S). Once in setup, find the entry for PCI slots and set these parameters (if they appear):

Slot Number:          		The slot where the adapter is installed (1-3)

Device:                			ENABLED

Master:                			ENABLED

Slave:                  			ENABLED

The exact wording of the parameters varies with different computers.

See chapter 5 in this document on how to install a 21X4 base board in

your system.

Some PCs do not implement bus mastering capabilities in all their PCI

slots. Please install your 21X4 based board in another slot as described

in chapter 5.

Message	DC21X4-DOS-210: FATAL: Unsupported BUS or missing BIOS

support.

Reason	The PC you are using does not have a PCI bus or that the BUS ID

keyword specified in NET.CFG contains a bus ID not supported by the

driver.

Action	Check that your PC has a PCI bus.

For CardBus adapter :

If you are using a static enabler, make sure that the enabler is loaded

prior to the loading of the driver.

If you are using a Card and Socket Services, make sure that the Card and

Socket Services were loaded prior to the loading of the driver.

Note: Card and Socket Services support is with DC21X4 v3.00 and above.

Check that the BUS ID keyword used in NET.CFG contains a supported

bus type (see BUS ID section in this document).

Message	DC21X4-DOS-211: FATAL: Failed to locate DC21X4 on board

Reason	The driver found an Ethernet controller in the system but it does not

contain a 21X4 controller in it.

Action	Make sure that the board installed in your PC contains a Intel

21X4 base device.

If this is a 21X4 based board, upgrade your DC21X4.COM driver.

Message	DC21X4-DOS-212: FATAL: Device is disabled, please check

your system bios configuration

Reason	The PCI slot where the 21x4 board is in, is disabled    in the bios setup.

Action	Enable the access to the slot in the bios setup.

Message	DC21X4-DOS-217: FATAL: Failed to find PCI BIOS

Reason	Your PC contains a bus, other than PCI, which is supported by the

DC21X4.COM driver but you have specified the “BUS ID    PCI    4” keyword in NET.CFG file.

Action	Please remove the ‘BUS ID PCI 4’    keyword from NET.CFG.

Message	DC21X4-DOS-218: FATAL: Bad PCI BIOS status

Reason	The driver received a fail status when calling one of the PCI BIOS

routines.

Action	Check if your system BIOS version is the most updated one and, if not,

please upgrade your BIOS.

Check for your system BIOS correctness using an external utility.

Message	DC21X4-DOS-219: FATAL: Bad PCI BIOS Register err

Reason	The driver received a fail status when calling one of the PCI BIOS

routines.

Action	Check if your system BIOS version is the most updated one and if not

please upgrade your BIOS.

Message	DC21X4-DOS-220: FATAL: PCI BIOS failed to initialize

DC21X4 CBIO reg!

Reason	PCI BIOS has initialized the DC21X4 I/O base address register, in the

Device PCI configuration space, with an illegal value (in the Intel x86

based machines this value should always be between 0 to FFFFHex).

Action	Enter to your PC BIOS setup utility,    find the entry for the PCI slots and

set these parameters (if they appear) in the slot where the 21X4 based

board    is located:

Slot Number:          	The slot where the adapter is installed (1-3)

Device:                		ENABLED

I/O address:          		Choose an I/O address (usually from a

predefined list).

The exact wording of the parameters varies with different computers.

See chapter 5 in this document on how to install a 21X4 base board in

your system.

If the above fails try to place your board in a different slot.

Check if your system BIOS version is the most updated one and if not

please upgrade your BIOS.

Message	DC21X4-DOS-222: WARNING: KWD LINE_SPEED Unsupported

speed - KWD Ignored

Reason	The user added the LINE_SPEED Keyword in NET.CFG with a

parameter value other than 10 or 100.

The keyword entry was ignored by the driver.

Action	Change the value associated with the LINE_SPEED keyword to either

10 or 100.

Message	DC21X4-DOS-222: WARNING: KWD <Keyword Name>  No param

Reason      	The user added the < Keyword    Name> in NET.CFG but did not specify

the parameter needed for this keyword.

The keyword entry was ignored by the driver.

Action	Please add a legal parameter to the KEYWORD as specified in the DC21X4

Keywords section of this document.

Message	DC21X4-DOS-222: WARNING: KWD <Keyword Name> Param too

small

Reason      	The value specified with the keyword < Keyword    Name> in NET.CFG is

too small.

The keyword entry was ignored by the driver.

Action	Please modify the parameter value for that KEYWORD to a valid one as

specified in the DC21X4 Keywords section of this document.

Message	DC21X4-DOS-222: WARNING: KWD <Keyword Name> Param too

big

Reason      	The value specified with the keyword < Keyword    Name> in NET.CFG is

too big.

The keyword entry was ignored by the driver.

Action	Please modify the parameter value for that keyword to a valid one as

specified in the DC21X4 Keywords section of this document, or remove the line entirely.

Message	DC21X4-DOS-225: FATAL: PCI BIOS allocated an illegal

Interrupt to DC21X4 Chip!

Reason	PCI BIOS has initialized the DC21X4 Interrupt register, in the

Device PCI configuration space, with an illegal value (in the Intel x86

single-processor DOS based machines this value should always be

between 0 to F Hex).

Action	Enter to your PC BIOS setup utility,    find the entry for the PCI slots and

set these parameters (if they appear) in the slot where the 21X4 based

board    is located:

Slot Number:          	      The slot where the adapter is installed (1-3)

Device:                		      ENABLED

Interrupt:          		      Choose an IRQ (usually from a predefined list).

Interrupt Edge-level:      Level

The exact wording of the parameters varies with different computers.

See chapter 5 in this document on how to install a 21X4 based board in

your system.

If the above fails, try to place your board in a different slot.

Check if your system BIOS version is the most updated one and, if not ,

please upgrade your BIOS.

Message	DC21X4-DOS-226: WARNING: Failed to handle Turbo switch

Reason	The DC21X4 cannot operate the 'Turbo' mechanism, as requested by the

user in NET.CFG, for one of the following reasons:

- The chipset on the motherboard does not support TURBO mode.

- The chipset parameters are unknown to the driver.

Action	You may remove the TURBO keyword from NET.CFG

Message	DC21X4-DOS-226: FATAL: Failed to handle Turbo switch

Reason	The driver call to the PCI BIOS in order to operate the 'Turbo' mechanism

returned a failure status.

Action	Please remove the TURBO keyword from NET.CFG and then reboot the

PC.

Message	DC21X4-DOS-227: FATAL: Internal CRC table corrupted

Reason	The DC21X4 Driver found an error in its internal CRC table either

because the driver file is corrupted or someone has changed the table's

contents.

Action	Please delete the DC21X4.COM driver installed in the system and

then reinstall a new copy of driver.

Message	DC21X4-DOS-228: WARNING: Could not handle Intel chipset

bug workaround successfully

Reason	The DC21X4 driver failed to operate chipset bug workaround because

a call to PCI BIOS returned a fail status.

Action	Check that you are using the most updated version of your system

BIOS, if not then upgrade your BIOS.

Reboot the system.

Message	DC21X4-DOS-229: WARNING: This machine Host to PCI

bridge has a known bug.

Driver limits DC21X4 bursts as a

workaround.

Reason	The DC21X4 driver discovered a Host to PCI chipset known to have a

bug which can be worked around by limiting the maximum burst size the 21X4 chip is allowed to perform in this system.

Action	None.

Message	DC21X4-DOS-230: FATAL: This machine Host to PCI bridge

has a known bug when using cache in WB

mode which may cause data corruption or

system hang Please change the cache

mode to WT (Write Through) as a

workaround.

Reason 	The DC21X4 driver discovered a Host to PCI chipset known to have a

bug when the system cache is programmed to work in Write Back mode. This bug can be worked around by switching the Cache operating mode from WB to WT (Write Through).

Action	Enter you PC BIOS setup utility and switch the Cache mode from WB

(Write Back) to WT (Write Through), save your changes and reboot your

system.

Message	DC21X4-DOS-231: WARNING: The PCI Host Bridge present in

this machine requires that the driver

switch to Store and Forward Mode.

Reason	The DC21X4 driver has detected a PCI Host Bridge with limited data

transfer rates present in the PC. The driver had modified the 21X4 chip

parameters for working    in such a system. This may slightly slow the

network performance.

Action	None.

Message	DC21X4-DOS-240: FATAL: Unable to locate Adapter ROM!

Reason	You are using a board having multiple 21X4 devices all sharing the same

ROM.

The DC21X4 driver failed to locate the device specific entry in that ROM.

Action	If you are using an early DC21X4.COM driver version, please upgrade

your driver.

Check if the 21X4 based board did not arrive with vendor specific drivers.

If it did then please use the vendor supplied driver.

If none of the above help call your Nic (board) vendor.

Message	DC21X4-DOS-240: FATAL: Failed to read ROM!

Reason	The DC21X4 driver has failed reading the serial ROM due to a either a

hardware    failure or corrupted data (in early generations of the 21X4

family of chips).

Action	Check your system PCI BIOS that the 21X4 device or associated slot are

not disabled.

If you are using an early DC21X4.COM driver version, please upgrade

your driver.

If none of the above helps call your Nic (board) vendor.

Message	DC21X4-DOS-240: FATAL: Unsupported ROM version

Reason	The version (and hence the format) of the data read from the

SROM is not supported by the driver.

Action	If you are using an early DC21X4.COM driver version, please upgrade

your driver.

Check if the 21X4 based board did not arrive with vendor specific drivers.

If it did then please use the vendor supplied driver.

If none of the above helps call your Nic (board) vendor.

Message	DC21X4-DOS-240: FATAL: This DC21X4 chip does not

support this ROM version.

Reason	The DC21X4 driver found that the format version of the data read

from    the SROM cannot be used with the 21X4 device being used.

Action	If you are using an early DC21X4.COM driver version, please upgrade

your driver.

Check if the 21X4 based board did not arrive with vendor specific drivers.

If it did then please use the vendor supplied driver.

If none of the above helps call your Nic (board) vendor.

Message	DC21X4-DOS-241: FATAL: Unsupported media selection

Reason	The DC21X4 driver found that the 21X4 based NIC or 21X4 device itself

does not support the requested media selection.

Action	Remove all media selection keywords from NET.CFG.

or

Select the AUTOSENSE keyword in NET.CFG.

or

Check the available media types on the NIC and use one of 	them as your

media keyword in NET.CFG

Message	DC21X4-DOS-241: FATAL: Could not find media or PHY to

work with

Reason	The DC21X4 driver failed to find any medium in the NIC.

This may happen due to bad Serial ROM programming or an MII PHY

failure.

Action	If you are using an external MII module please insert the module and

reboot the system.

Try rebooting the system either by using the RESET switch or by

turning the Power off and then On again.

If you are using an early DC21X4.COM driver version, please upgrade

your driver.

Check if the 21X4 based board did not arrive with vendor specific drivers,

If it did then please use the vendor supplied driver.

If none of the above helps call your Nic (board) vendor.

Message	DC21X4-DOS-241: FATAL: The Error routine had been

called.

Reason	 The DC21X4 driver called an uninitialized internal routine entry.

Action	If you are using an early DC21X4.COM driver version, please upgrade

your driver.

If you are using the most updated driver then call your Nic vendor.

Message	DC21X4-DOS-242:  WARNING: Could not find a connected

medium

Reason	The Power-Up autosense activity performed by the driver failed to

discover a cable connected to one of the NIC’s media ports.

Action	If you are using a 21040 or 21140 based NICs then please connect a

cable to one of the ports, unload the driver and then reload it again.

If you are using other 21X4 device then please connect a cable to one of

the ports and the Autosense routine    will automatically find and use the

appropriate medium.

Message	DC21X4-DOS-244:  FATAL: Failed to register with Card

Services.

Reason	The driver cannot register itself with this version of Card Services.

Action	Please check that the driver supports this version of Card Services.

## 6.5 	DOS ODI Driver Counters

The Novell NetWare DOS ODI driver counters are divided into two groups. The first are counters defined by Novell which contain network management information and are implemented and used according to Novell NetWare HSM 4.0 spec guidelines.

The other is a group of driver specific counters which may contain any information thought to be helpful by the DC21X4 driver developer.

These counters can be viewed by Novell DOSTEST utility.

A short explanation of each counter is given in the following two subsections.

#### 6.5.1 	General DOS ODI Driver Counters

##### MTotalTxPackets

The total number of packets sent to the driver for transmission by the protocol stack.

##### MTotalRxPackets

The total number of packets received from the network by the 21X4 chip and driver which were consumed by the protocol stack.

##### MNoECBsAvailable

The total number of packets received from the network by the 21X4 chip and driver which the protocol stack refused to accept.

##### MTxTooBig

The total number of packets sent to the driver for transmission by the protocol stack that were too big to send.

Ethernet networks should not transmit frames bigger than 1,518 bytes.

##### MTxTooSmall

The total number of packets sent to the driver for transmission by the protocol stack that were too small to send as is.

Ethernet networks should not transmit frames smaller than 64 bytes.

Usually these packets are transmitted with padding.

##### MRxOverflow

The total number of packets received from the network by the 21X4 chip which could not be accepted by the driver due to lack of internal buffers.

##### MRxTooBig

The number of incoming packets that were bigger than 1,518 bytes long.

Ethernet networks should not carry frames bigger than 1,518 bytes.

##### MRxTooSmall

The number of incoming packets that were smaller than 64 bytes long.

Ethernet networks should not carry frames smaller than 64 bytes.

##### MTxMiscError

The number of transmission requests that were not sent because of errors other than those explicitly listed in this section.

This counter is incremented whenever a driver specific transmit error counter is incremented.

##### MRxMiscError

The number of incoming packets that were lost because of errors other than those explicitly listed in this section.

This counter is incremented whenever a driver specific receive error counter is incremented.

##### MTxRetryCount

The number of times the driver had to retransmit the packet because of hardware errors.

This counter is not being used by the driver.

##### MRxChecksumError

The total number of incoming packets which were lost due to checksum/CRC errors.

##### MRxMismatchError

The total number of incoming packets which were lost due to conflicting information given by the hardware and the media specific header.

##### MQueueDepth

The number of packets queued in the driver for transmission.

#### 6.5.2 	Custom DOS ODI Driver Counters

##### Tx Watchdog

Indicates that the 21X4 transmitter is jabbering the network.

This will cause DC21X4 driver to shut down the hardware and halt itself ignoring any request from the protocol stack.

You will need to reboot the system in order to resume work.

##### Tx Heart Beat Fail

The total number of    packets transmitted, in which the post transmission heartbeat collision check has failed.

This check is meaningful when using an AUI medium only and does not necessarily indicate that the transmitted packet was not sent.

There are quite a few AUI Hubs in the market which do not implement this check.

##### Tx TP Link Fail

The number of packets which failed to transmit due to a Link check failure.

##### Tx Initially Deferred

The number of times that the 21X4 chip had to defer while ready to transmit a frame because the carrier was asserted.

This situation is treated as a collision by the 21X4 chip.

##### Tx Single Collision

The number of packets which were successfully transmitted after one 21X4 chip trial to access the network.

##### Tx Multiple Collisions

The number of packets which were successfully transmitted after more than one 21X4 chip trial to access the network.

##### Tx FIFO Underflow

The number of packets which the 21X4 had to abort their transmission because the data arrived late from memory.

This usually indicates poor system performance; if this counter increments rapidly then use the TX_THRESHOLD with value of 4. This will overcom the problem.

##### Tx Late Collision

The number of packets which the 21X4 had to abort their transmission because a collision occurred after the maximum collision window of 64 bytes.

This may indicate that an Ethernet controller connected to the network or a network cable length which violate the Ethernet spec.

##### Tx Loss or no Carrier

The number of packets which the network Carrier was lost or not present    when the 21X4 device transmitted them.

##### Tx Excessive Collisions

The number of packets the 21X4 device aborted their transmission after receiving 16 successive collisions when attempting to transmit them.

##### Rx Late Collision

The number of packets received with a damaged frame due to a late collision. This may indicate that an Ethernet controller connected to the network or a network cable length which violate the Ethernet spec.

Late collision is a collision occurring after the legal 64 byte collision window.

##### Rx Watchdog

The total number of packets which    their reception was stopped by the 21X4 chip since the packet length exceeded 2048 bytes.

Ethernet networks should not contain frames bigger than 1,518 bytes.

##### Rx Alignment Error

The total number of packets received with both CRC error and dribbling bits indication.

##### Rx FIFO Overflow

The total number of packets the 21X4 device had sensed on the network which could not be receive by the 21X4 device itself due to lack of space in its internal receive FIFO.

This may indicate a slow system bus which prevents the chip from transferring its internal FIFO contents to the system memory.

Increasing the PBL using the PBL keyword or using the TUBO keyword may improve this situation.

##### RX ECB Too Small

The number of packets which their data transfer to the protocol stack was aborted since the buffer provided by the protocol stack    was too small.

##### Num Restarts DC21X4

The number of times the driver had reset the 21X4 device.

##### DMC Out Of Resources

This counter is not being used by the DC21X4.COM driver v2.40 or above.

##### Fatal System Error

Indicates that the 21X4 device had identified one of the following    PCI system errors

Parity error, Master abort or Target abort. This is a Fatal error and the system may be in an undefined state.

This will result the DC21X4 driver to shut down the hardware and halt itself ignoring any request from the protocol stack.

You will need to reboot the system in order to resume work.

##### Driver is halted

Indicates that the DC21X4 driver had shut down the 21X4 hardware and itself ignoring any request from the protocol stack from this time on.

You will need to reboot the system in order to resume work.

- APPENDIX A:    Novell NetWare Environment.

Note:	The information listed below is subject to change by the respective companies.

## Major Software Components

A Novell Local Area Network (LAN) has two major software components:

- NetWare Operating system

- Workstation software.

### NetWare    Operating System

The NetWare operating system resides in the file server and controls the shared file server devices.

### Workstation Software

The local operating system (typically DOS, MS-Windows or OS/2) resides at the workstation.

Acting as a traffic redirector between NetWare and the local operating system is a software program called the shell (DOS or MS-Windows 3x    machines) or requester (OS/2    and MS-Windows 9x, NT machines). This program resides at the workstation.

The shell/requester intercepts each request made by the user and directs it to either the file server or the local operating system, depending on the type of request.

#### NetWare Shell

the NETx.COM or VLM programs contain the NetWare shell. The shell lies between the application layer and DOS, monitoring all data    transmission that moves in or out either one.

The NetWare shell uses IPX to achieve communications. Important points are as follows:

- The shell uses IPX program (IPXODI.COM) to assign source and destination addresses to a data packet.

- IPX uses a LAN driver to control transmissions through the network board. (The LAN driver is a piece of software that interfaces between the operating system or shell and the board; The DC21X4.COM is such a LAN driver).

#### Open Data Link Interface (ODI)

The ODI allows multiple network protocols and LAN adapters to be used concurrently on the same workstation or file server.

It provides a flexible, high-performance Data Link Layer interface to Network Layer protocol stacks. The low-level    communications subsystem is comprised of three elements

- Protocol Stacks

- Link Support Layer (LSL)

- Multiple Link Interface Driver (MLIDS)

##### Multiple Link Interface Driver (MLIDS)

MLIDs are device drivers that handle the sending and receiving of packets to and from a physical or logical LAN medium. MLIDs interface with a Network Interface Card (NIC or LAN adapter) and handle frame header appending and stripping. They also help demultiplex the incoming packets by determining the packet’s frame format.

Each MLID’s interface with the physical board is determined by the board’s hardware.

All MLIDs can handle packets from various protocols because the MLID does not interpret the packet. Instead it passes received packets to the Link Support Layer (LSL)

using Event Control Blocks (ECBs), structures used to send or receive packets.

The DC21X4.COM is am example of a MLID driver.

##### Link Support Layer (LSL)

The LSL handles the communication between protocol stacks and MLIDs. Because the ODI allows the physical LAN medium to support many different types of protocols (for example, IPX, TCP/IP, AppleTalk and LAT might all be supported on one Ethernet network), the MLID receives packets destined for different protocol stacks that might be present in the system. The LSL then determines which protocol stack id to receive the    packet. Next, the protocol stack determines what should be done with the packet and where it should be sent. When the protocol stack must transmit a packet, the protocol stack hands the packet to the LSL, which then routs the packet to the appropriate MLID.

The LSL also tracks the various protocol and MLIDs that are currently loaded in the system and provides a consistent method of finding and using each of the loaded modules.

The LSL also performs the following services:

- Allows a protocol stack to obtain and return ECBs.

- Queues and recovers ECBs for later use.

- Registers and deregisters the stack

- Allows protocol stacks to determine Stack and Protocol Ids.

- Allows protocol stacks to obtain MLID statistics.

- Allows protocol stacks to bind with MLIDs

- Allows protocol stacks to transmit and receive packets through an MLID.

- Maintains a list of all active stacks and MLIDs.

- Allows protocol stacks to obtain information about MLIDs and other protocol stacks.

##### Protocol Stacks

Network Layer protocol stacks transmit and receive data over a logical or physical network. They also handle routing, connection services and APIs, and provide an interface to allow higher layer protocols or applications access to the protocol stack’s services.

#### Drive Mappings

Drive pointers are letters assigned by the operating system to reference different areas where files can be stored. Both DOS and NetWare use drive pointers to move quickly to these areas.

##### DOS Drive Pointers

DOS pointers reference local devices in a workstation by assigning them a letter of the alphabet known as a drive pointer. To move to the different drives, you type the letter assigned (followed by a colon) to that physical device.

Once you are on the physical drive, the DOS CD command allows you to move between directories on the drive.

##### NetWare Drive Pointers

NetWare also uses letters of the alphabet as drive pointers. However, NetWare uses them to identify logical locations within the system’s directory structure, not just predefined physical locations.

###### Drive Pointer Types

Directories which store data files, and directories which contain executable files such as applications, are kept separate on the NetWare directory structure. The two types of directories are likewise referenced by different kinds of NetWare drive pointers

- Network

- Search

###### Network Drive Assignments

- Network Drive Pointers

A network drive letter is assigned to reference directories where user-generated files are kept. This includes directories which store database records or other user-created files.

- Network Drive Letters

There are 26 letters available to each workstation to assign as regular drive pointers.

Depending on the local operating system being used, some of the drive pointers are used to represent locations on local devices. For instance DOS by default uses driver A-E. the remaining letters may be assigned as network drives to point to various locations within the directory structure.

###### Search Drive Assignments

- Search Drive Pointers

A search drive allows a user, located in a data directory, to access executable files that are located in another directory.

- Search Drives

Search drives are numbered; they correspond to network drives in reverse alphabetical order. NetWare allows each user to have up to 16 search drives. Search drive pointers are used to tell NetWare which directories, and in what order, to search for a file.

###### Drive Pointer Assignments (MAP)

The MAP utility assigns network drive pointers to directories.

- APPENDIX B: Hotline and Support Information.

Important Notice

As of May 17, 1998, Digital Equipment Corporation's StrongARM,

PCI Bridge, and Networking component businesses, along with the chip

fabrication facility in Hudson, Massachusetts, were acquired by

Intel Corporation and transferred to Intel Massachusetts, Inc.

As a result of this transaction, certain references to web sites,

telephone numbers, and fax numbers have changed in the documentation.

This information will be updated in the next version of this manual.

Copies of documents that have an ordering number and are referenced

in this document, or other Intel literature may be obtained by calling

1-800-332-2717 or by visiting Intel's website for developers at:

http://developer.intel.com

The Intel Massachusetts Customer Technology Center continues to

service Network Product technical inquiries. Please use the

following information lines for support:

For documentation and general information:

Intel Massachusetts Information Line

United States: 1-800-332-2717

Outside United States: +1-303-675-2148

Electronic mail address: techdoc@intel.com

For technical support:

Intel Massachusetts Customer Technology Center

Phone (U.S. and international): 1-978-568-7474

Fax: 1-978-568-6698

Electronic mail address: techsup@intel.com

Upgrading Drivers

See the components page available on the Intel World Wide Web

Internet site at:

http://developer.intel.com/design/network/new21/download/dsc-software-nc.htm